ANNEX 1 – Technical Specifications

# Cable Reel Trailers

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 3

Delivery Location = Ebeye Port, Kwajalein Atoll, Marshall Islands – Qty = 3

Technical Specifications for Cable reel trailers

| Item | Specification | Basic Details |
| --- | --- | --- |
|  | Vehicle Type | Cable Reel Trailers |
|  | Type | Rubber tired self loading cable drum trailer |
|  | Drum Capacity | Minimum diameter 2.5 meters (99”) width 1.5 meters (60 “) |
|  | Carrying Capacity | Up to 3 tons (6000 lbs) |
|  | Loading/Unloading | Self Loading manual Hydraulic or mechanical drum lifting system |
|  | Braking System | Air brake system or hydraulic brake provide options |
|  | Frame | Galvanized steel chassis; stress-relieved and reinforced |
|  | Lighting | Tail lights and reflectors compliant with road regulations |
|  | Suspension | Leaf spring or torsion axle rated for local terrain |
|  | Brake Type | Inertia brake or dual air system with parking lock |
|  | Hitch | Pintle |
|  | Towing Compatibility | Pin-type drawbar with adjustable height range 16-24 inches (400-610mm) from ground to center of pin, optimized for coupling with standard 4x4 utility vehicles. Drawbar assembly shall include safety chains, breakaway cable provision, and all necessary hardware for secure coupling. |
|  | Standards | SAE J560 electrical connectors; DOT road safety compliance |
| Environmental and Operational Conditions |  |  |
|  | Temperature | Designed to operate in high ambient temperatures up to 50°C |
|  | Humidity | Suitable for environments with relative humidity up to 98% |
|  | Corrosion Protection | All exposed metallic parts shall be hot-dip galvanized or stainless steel to resist rust and corrosion |
|  | Coatings | Anti-corrosive primer and UV-resistant paint for prolonged exposure to coastal conditions |
|  | Sealing | Electrical and control compartments should meet IP65 minimum rating for water and dust protection |
|  | Test Certificate | Including: - Structural Tests: Inspecting the frame, and other critical components for structural integrity. This may involve visual inspections and potentially load tests. - Functional and Operational Tests: Verifying that all controls, hydraulics, and safety systems function correctly. - Documentation and Sign-Off: The Test Certificate should be well-documented, including a punch list of any issues found and a final sign-off from both the manufacturer and the customer. |
|  | Documentation | Manual on vehicle standard operating procedures (SOPs) |
| Accessories and Documentation |  |  |
|  | Drum shaft with adjustable width |  |
|  | Operator’s manual |  |
|  | Safety chains and wheel chocks |  |
|  | Spare parts list for 2 years |  |
|  | Certificate of roadworthiness |  |
|  | Mud Guards & Flaps |  |
|  | Warning and safety decals |  |
|  | Requirement is for Two Sets of 3 units (our location requirements are not suitable for a single three reel trailer system. Three separate trailers will be easier to position as needed). |  |

Truck Cranes

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 2

Delivery Location = Ebeye Port, Kwajalein Atoll, Marshall Islands – Qty = 1

Technical Specifications for Truck Cranes

| Item | Specification | Basic Details |  |
| --- | --- | --- | --- |
|  | Vehicle Type | Cranes (Truck-Mounted Hydraulic Crane) |  |
|  | Truck Cab | Air conditioning Full mirrors Side steps Heavy duty seat covers Left Hand Drive Back up Camera |  |
|  | Truck bed dimensions | Standard truck bed dimensions. Must include an integrated and removable boom rest on the bed. |  |
|  | Truck bed load capacity |  Minimum bed load capacity: 10,000 lbs (5 tons) when crane is not in use, with load evenly distributed  Minimum bed load capacity during crane operations: 5,000 lbs (2.5 tons) with crane fully extended and loaded to rated capacity Bed surface to be min 3/8” checker plate steel Bed to have stake pockets on the sides and rear Bed to incorporate ratchet roller load binder straps min of 4” wide Under deck compartment for chains, ropes, straps and slings |  |
|  | Structure for carrying poles | The truck must be equipped with a structure that allows to carry one or two long poles. Max length of pole is 50 feet long but common length is 40 feet. |  |
|  | Crane Operator Position | Behind truck cab, protected by deck wall. Dual operator stations each side of truck |  |
|  | Maximum Lifting Capacity: | Min lifting capacity shall be 25 Tons Weight load indicator – Digital and viewed from both operator positions |  |
|  | Boom Length and Reach | Must be capable of 70 feet reach of main telescoping boom (excluding optional attachment booms) |  |
|  | 360° Slewing | Continuous rotation with hydraulic slew drive |  |
|  | Maximum Lift Height: | (Must be verified with MEC, see definition below) Required lift height is 70 feet but this does not need to be the maximum. |  |
|  | Hooks | All hooks to include safety latching clips Include single line headache ball Include two part pulley block hook Include spare keys and wedges |  |
|  | Mounting | Mounted on a 4x4 or 6x6 truck chassis suitable for off-road and utility use |  |
|  | Engine Type | Diesel Automatic transmission preferred |  |
|  | Control | Dual control (manual and remote if possible) |  |
|  | Stabilizers | Hydraulic outriggers for stabilization |  |
|  | Power Take Off (PTO) | Hydraulic PTO |  |
|  | Tool Storage | Lockable tools and straps/chains boxes each side |  |
|  | Towing | 15 Ton Pintle combo towing hitch with 2” ball Trailer brake controls Trailer power connection Trailer Air connections |  |
|  | Corrosion Protection | Complete undersealing of vehicle |  |
|  | Spares (per vehicle) | Fuel filters (all types) – 12 spares Engine Oil filters – 12 spares Hydraulic Oil filters – 12 spares Air filters 12 spare All Belts – 2 sets for all One roll of replacement hoist rope |  |
|  | Safety Features | Load moment indicator (LMI), emergency stop, anti-two block device Reflective Decals LED work spotlights and floodlights Rooftop Emergency Service Strobe LED Light bar Backup Alarm Horn and camera Heavy Duty Front Crash bars – full body width Front and rear mudflaps Rubber Wheel chocks Level Indicators |  |
|  | Special Fluids | DEF (if Required) must include volume for min 20,000 miles |  |
|  | Certifications | ISO 9001 manufacturing / EN 12999 Crane safety |  |
| Environmental Conditions |  |  |  |  |
|  | Operational temperature range | -10°C to +45°C |  |
|  | Humidity | Suitable for environments with relative humidity up to 98% |  |
|  | Altitude | Up to 1,000 m above sea level |  |
|  | Weather Resistance | All-weather operation with corrosion-resistant components |  |
|  | Corrosion Protection | All exposed metallic parts shall be hot-dip galvanized or stainless steel to resist rust and corrosion Full body undercoating is required |  |
|  | Coatings | Anti-corrosive primer and UV-resistant paint for prolonged exposure to coastal conditions |  |
|  | Sealing | Electrical and control compartments should meet IP65 minimum rating for water and dust protection |  |
|  | Diesel Fuel Spec | Vehicle must be able to run on the fuel specification below; - Automotive Diesel Fuel (50 ppm S) - Cetane Index min of 46 per ASTM D976 - Density @ 15 degs C 820.0 – 860.0 Kg/M3 per ASTM D4052 - Flash Point – PMCC min of 64.0 degs C per ASTM D93 - Viscosity @ 40 degs C of 2.0 – 4.5 CST as per ASTM D445 - Lubricity, max of 460 Micron as per IP450 - Full specification available upon request. |  |
|  | Training | At least two days of in-country training for six employees on SOPs, equipment, safety, and maintenance |  |
|  | Factory Acceptance Test (FAT) | Including: - Structural Tests: Inspecting the frame, boom, outriggers, and other critical components for structural integrity. This may involve visual inspections and potentially load tests. - Functional and Operational Tests: Verifying that all controls, hydraulics, and safety systems function correctly. This includes checking the crane's movement, outriggers, brakes, lights, and alarms. - Documentation and Sign-Off: The FAT should be well-documented, including a punch list of any issues found and a final sign-off from both the manufacturer and the customer. |  |
|  | Lifting Charts | The supplier must provide certified Lifting Charts. |  |
|  | Documentation | Manual on vehicle standard operating procedures (SOPs) |  |
| Accessories and Documentation |  |  |  |  |
|  | Operator’s manual |  |  |
|  | Maintenance manual |  |  |
|  | Spare parts for 2 years |  |  |
|  | Tool kit for routine service |  |  |
|  | Supplier shall provide certified load rating chart showing safe bed load capacities at various crane positions and loads. Maximum combined weight (crane load + bed load) shall not exceed vehicle Gross Vehicle Weight Rating (GVWR). |  |  |

Mobile Test Rigs

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 1

Delivery Location = Ebeye Port, Kwajalein Atoll, Marshall Islands – Qty = 1

Technical Specifications for Mobile test rigs

| Item | Specification | Basic Details |
| --- | --- | --- |
|  | Vehicle Type | The Mobile Test Rig (for Underground Cables) requires a heavy-duty waterproof cover made of UV-resistant synthetic material (such as polyester with PVC/polyurethane coating or marine-grade reinforced vinyl) to protect the equipment while in storage and during periods of non-use, as well as from sudden rain showers when deployed in the field. The cover material shall be rot-resistant, mildew-resistant, and rated for tropical marine environments with minimum 600 denier fabric strength. The unit should have an IP66 rating for protection against dust and water. |
|  | Electric utility frequency | 60 Hz |
|  | Test Voltage Capability | VLF test voltage range from 0.1 Hz up to 80 kV peak |
|  | Measurement Functions | - VLF withstand and tan delta measurement - Insulation Resistance (IR) up to 5 GΩ - Fault location (Time Domain Reflectometry + Surge generator) - Cable fault locator |
|  | Power Options | Diesel generator (3–5 kVA) + 120V AC input |
|  | Environmental Protection | IP54-rated enclosure for test modules |
|  | Software Integration | Windows-based logging system with USB/Cloud backup |
|  | Certifications | IEC 60229, IEEE 400.2, IEC 60502 |
| Environmental and Operational Conditions |  |  |
|  | Temperature | Designed to operate in high ambient temperatures up to 50°C |
|  | Humidity | Suitable for environments with relative humidity up to 98% |
|  | Corrosion Protection | All exposed metallic parts shall be hot-dip galvanized or stainless steel to resist rust and corrosion |
|  | Coatings | Anti-corrosive primer and UV-resistant paint for prolonged exposure to coastal conditions |
|  | Sealing | Electrical and control compartments should meet IP65 minimum rating for water and dust protection |
|  | Training | At least two days of in-country training for six employees on SOPs, equipment, safety, and maintenance |
|  | Test Certificate | Including: - Functional and Operational Tests: Verifying that all controls and safety systems function correctly. brakes, lights, and alarms. - Dielectric Tests: Assessing the electrical insulation - Documentation and Sign-Off: The Test Certificate should be well-documented, including a punch list of any issues found and a final sign-off from both the manufacturer and the customer. |
|  | Documentation | Manual on vehicle standard operating procedures (SOPs) |
| Accessories and Documentation |  |  |
|  | Calibration certificates |  |
|  | Software for report generation |  |
|  | Training session for technicians |  |
|  | Spare parts list for 2 years |  |
|  | Complete user and service manual |  |

Service Vehicles

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 4

Delivery Location = Ebeye Port, Kwajalein Atoll, Marshall Islands – Qty = 2

Technical Specifications for Inspection and Service Vehicles

| Item | Specification | Basic Details |
| --- | --- | --- |
|  | Vehicle Type | 4x4 pickup truck, or van suitable for Utility lineman service vehicle |
|  | Number of doors for passengers | 4 doors crew cab |
|  | Drive | LHD Left Hand Drive |
|  | Transmission | Automatic |
|  | Color | White |
|  | Seating | Minimum 2 front + 3 rear seats |
|  | Payload | Minimum 1,000 kg payload capacity |
|  | Equipment | - Ladder rack - Lockable tool storage compartments - First aid kit - Fire extinguisher |
|  | Electrical Equipment | Dual battery system, 12V–120V inverter (1.5–2.0 kW) Dual waterproof power outlets on each side of utility body |
|  | Safety | ABS, 2 airbags, rearview camera, roll bar, reflective decals Roof top emergency services strobe light bar Back up alarm horn Front crash bars, full body width |
|  | Tool Storage | Lockable aluminum compartments; IP65-ratedd |
|  | Certifications | ISO 7637-2 (Electrical equipment in vehicles); ADR compliant |
|  | Utility Body Build | Rust-proof aluminum or fiberglass canopy with LED lighting |
|  | Towing | Pintle combo two hitch integrated into rear bumper, min 4000lb towing capacity 7 pin electrical connector for trailer electrical |
|  | Brand Integration | Fit-out by certified upfitters like Adrian Steel, Sortimo, Rhino, Bott, Sortimo, Auto Transform or an equivalent company approved by the manufacturer |
|  | Engine System | Preference is for diesel engine |
|  | Diesel Fuel Spec | Vehicle must be able to run on the fuel specification below; - Automotive Diesel Fuel (50 ppm S) - Cetane Index min of 46 per ASTM D976 - Density @ 15 degs C 820.0 – 860.0 Kg/M3 per ASTM D4052 - Flash Point – PMCC min of 64.0 degs C per ASTM D93 - Viscosity @ 40 degs C of 2.0 – 4.5 CST as per ASTM D445 - Lubricity, max of 460 Micron as per IP450 - Full specification available upon request. |
| Safety and Compliance |  |  |
|  | Temperature | Designed to operate in high ambient temperatures up to 50°C |
|  | Humidity | Suitable for environments with relative humidity up to 98% |
|  | Corrosion Protection | All exposed metallic parts shall be hot-dip galvanized or stainless steel to resist rust and corrosion Body undercoating system |
|  | Coatings | Anti-corrosive primer and UV-resistant paint for prolonged exposure to coastal conditions |
|  | Documentation | Manual on standard vehicle operating procedures (SOPs) |
| Documentation |  |  |
|  | Owner’s manual, parts manual |  |
|  | Service schedule with details of service requirements |  |
|  | Full size Spare tire plus jack and tool kit |  |
|  | Warranty (minimum 2 years or 50,000 km) |  |

Tracked Vehicles

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 2

Delivery Location = Ebeye Port, Kwajalein Atoll, Marshall Islands – Qty = 1

Technical Specifications for Mini Excavator - Tracked Vehicles

| Item | Specification | Basic Details |
| --- | --- | --- |
|  | Vehicle Type | Mini Excavator - tracked |
|  | Cab | ROPS Enclosed operator cab with Air conditioning, wiper, seatbelt, USB ports (min 2) Travel pedals and hand levers Color monitor Cup holder |
|  | Guarding | Heavy duty front guard ISO 10262:1998 Level II Heavy Duty Top guard ISO 10262:1998 Level II |
|  | Digging Depth | 126 in |
|  | Boom, Stick, Linkages | 2nd Auxiliary hydraulic lines Bucket divert lines Boom lowering check valve Stick lowering check valve Hydraulic quick coupler lines Thumb ready Certified lifting eye |
|  | Backhoe bucket | 12” HD, 18” HD, 24”HD, 48” grading bucket, Thumb |
|  | Extendable Digging stick | Depths diggable to |
|  | Digging force | Standard stick 4721 lbf Bucket 8588 lbf |
|  | Undercarriage | Dozer straight blade Dozer blade float Tiedowns on track frame |
|  | Controls | Adjustable wrist pads |
|  | Tracks | Steel tracks with rubber pads |
|  | Hydraulic system | Variable Displacement piston pump Load sensing/flow sharing hydraulics Hydraulic oil cooler Accumulator Auxiliary Hydraulic lines Continuous Auxiliary flow One and two way Auxiliary flow |
|  | Operating weight | 9500lb+ |
|  | Ground clearance | Min 12” |
|  | Ground pressure | At max weight 4.9psi |
|  | Equipment | - First aid kit - Fire extinguisher - External tool box - Transport tie downs |
|  | Electrical Equipment | Heavy duty battery system Display screen , soft key |
|  | Safety | rearview camera, roll bar, Signal/warning horn reflective decals Roof top emergency services strobe light bar Beacon - rotating and strobe LED Front & Rear lights LED Boom lights |
|  | Tool attachments | Hydraulic attachment quick coupler Ripper Vibratory compaction plate Hammer |
|  | Engine System | diesel engine 45Hp+ Fuel Water separator |
|  | Diesel Fuel Spec | Vehicle must be able to run on the fuel specification below; - Automotive Diesel Fuel (50 ppm S) - Cetane Index min of 46 per ASTM D976 - Density @ 15 degs C 820.0 – 860.0 Kg/M3 per ASTM D4052 - Flash Point – PMCC min of 64.0 degs C per ASTM D93 - Viscosity @ 40 degs C of 2.0 – 4.5 CST as per ASTM D445 - Lubricity, max of 460 Micron as per IP450 - Full specification available upon request. |
| Safety and Compliance |  |  |
|  | Temperature | Designed to operate in high ambient temperatures up to 50°C |
|  | Humidity | Suitable for environments with relative humidity up to 98% |
|  | Corrosion Protection | Under body coating protection system |
|  | Coatings | Anti-corrosive primer and UV-resistant paint for prolonged exposure to coastal conditions |
|  | Documentation | Manual on standard vehicle operating procedures (SOPs) |
| Documentation |  |  |
|  | Owner’s manual, parts manual |  |
|  | Service schedule with details of service requirements |  |
|  | Warranty (minimum 2 years) |  |

Loader Backhoe Vehicles

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 1

Delivery Location = Ebeye Port, Kwajalein Atoll, Marshall Islands – Qty = 1

Technical Specifications for Loader Backhoe Vehicles

| Item | Specification | Basic Details |
| --- | --- | --- |
|  | Vehicle Type | 4x4 Loader Backhoe AWD |
|  | Cab | ROPS Enclosed operator cab with Air conditioning, wiper, seatbelt, USB ports (min 2) |
|  | Drive | Center steering position |
|  | Transmission | Automatic |
|  | Color | Any |
|  | Seating | One operator |
|  | Payload | Minimum 10,000lb payload capacity |
|  | Loader bucket | 1.5 cu yds |
|  | Backhoe bucket | 12” HD, 18” HD, 24”HD, 48” grading bucket, |
|  | Extendable Digging stick | Depths digable to 5748mm |
|  | Breakout force | 13646 lbf |
|  | Controls | Pilot Hoe joysticks Pattern Selector (ISO/SAE) |
|  | Steering | All wheel steering Hydraulic steering 100% differential lock Front fenders |
|  | Tires | 28” Tires, front and back tires are same size |
|  | Wheelbase | Minimum 72 inches |
|  | Hydraulic system | Variable Displacement piston pump Side shift lock Hydraulic oil cooler Loader kickout, float Loader kickout, return to dig Powered side shift Ride Control Continuous flow Auxiliary tool lines and pump Programable Auxiliary flow |
|  | Operating weight | 20000lb+ |
|  | Ground clearance | Min 18 inches measured at lowest point with standard bucket/attachments |
|  | Equipment | - First aid kit - Fire extinguisher - External tool box - Transport tie downs |
|  | Electrical Equipment | Dual heavy duty battery system Display screen , soft key |
|  | Safety | rearview camera, roll bar, reflective decals Roof top emergency services strobe light bar Back up alarm horn Stabilizer alarm Load stability indicator Beacon - rotating and strobe Road lights |
|  | Tool attachments | Hydraulic attachment quick coupler Ripper Vibratory compaction plate Hammer |
|  | Skid plates | Under belly skid and rock protection plates |
|  | Engine System | diesel engine 90Hp+ Hinged radiator pack Fuel Water separator |
|  | Diesel Fuel Spec | Vehicle must be able to run on the fuel specification below; - Automotive Diesel Fuel (50 ppm S) - Cetane Index min of 46 per ASTM D976 - Density @ 15 degs C 820.0 – 860.0 Kg/M3 per ASTM D4052 - Flash Point – PMCC min of 64.0 degs C per ASTM D93 - Viscosity @ 40 degs C of 2.0 – 4.5 CST as per ASTM D445 - Lubricity, max of 460 Micron as per IP450 - Full specification available upon request. |
| Safety and Compliance |  |  |
|  | Temperature | Designed to operate in high ambient temperatures up to 50°C |
|  | Humidity | Suitable for environments with relative humidity up to 98% |
|  | Corrosion Protection | Under body coating protection system |
|  | Coatings | Anti-corrosive primer and UV-resistant paint for prolonged exposure to coastal conditions |
|  | Documentation | Manual on standard vehicle operating procedures (SOPs) |
| Documentation |  |  |
|  | Owner’s manual, parts manual |  |
|  | Service schedule with details of service requirements |  |
|  | Warranty (minimum 2 years) |  |

Telehandler Vehicles

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 2

Technical Specifications for Telehandler Vehicles

| Item | Specification | Basic Details |
| --- | --- | --- |
|  | Vehicle Type | 4x4 telescopic forklift |
|  | Cab | Enclosed operator cab with Air conditioning, wiper, seatbelt, USB ports (min 2) |
|  | Drive | LHD Left Hand Drive |
|  | Transmission | Automatic |
|  | Color |  |
|  | Seating | One operator |
|  | Payload | Minimum 10,000lb payload capacity |
|  | Max lift height | 50+ feet |
|  | Forward reach | 40+ feet |
|  | Frame levelling | 10 degrees |
|  | Drawbar pull loaded | 29,000lbf |
|  | Capacity at max lift height (stabilizers down) | 5000lbs |
|  | Capacity at max forward reach (stabilizers down) | 3000lbs |
|  | Operating weight | 29000lb |
|  | Ground clearance | Min 18” |
|  | Equipment | - First aid kit - Fire extinguisher |
|  | Electrical Equipment | Dual heavy duty battery system |
|  | Safety | rearview camera, roll bar, reflective decals Roof top emergency services strobe light bar Back up alarm horn Load stability indicator Window guards front and top Beacon Road lights |
|  | Tool attachments | 48” pallet forks 72” pallet forks General purpose bucket Hydraulic attachment coupler |
|  | Towing | Retrieval Hitch and rear pintle hitch |
|  | Engine System | diesel engine 70Hp+ |
|  | Diesel Fuel Spec | Vehicle must be able to run on the fuel specification below; - Automotive Diesel Fuel (50 ppm S) - Cetane Index min of 46 per ASTM D976 - Density @ 15 degs C 820.0 – 860.0 Kg/M3 per ASTM D4052 - Flash Point – PMCC min of 64.0 degs C per ASTM D93 - Viscosity @ 40 degs C of 2.0 – 4.5 CST as per ASTM D445 - Lubricity, max of 460 Micron as per IP450 - Full specification available upon request. |
| Safety and Compliance |  |  |
|  | Temperature | Designed to operate in high ambient temperatures up to 50°C |
|  | Humidity | Suitable for environments with relative humidity up to 98% |
|  | Corrosion Protection | All exposed metallic parts shall be hot-dip galvanized or stainless steel to resist rust and corrosion Body undercoating system |
|  | Coatings | Anti-corrosive primer and UV-resistant paint for prolonged exposure to coastal conditions |
|  | Documentation | Manual on standard vehicle operating procedures (SOPs) |
| Documentation |  |  |
|  | Owner’s manual, parts manual |  |
|  | Service schedule with details of service requirements |  |
|  | Full size Spare tire plus jack and tool kit |  |
|  | Warranty (minimum 2 years or 50,000 km) |  |

Bucket Truck - Small

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 2

Delivery Location = Ebeye Port, Kwajalien Atoll, Marshall Islands – Qty = 1

Technical Specifications for Bucket Truck - Small

| Item |  | Specification | Basic Details |
| --- | --- | --- | --- |
|  |  | Vehicle Type | Utility Bucket Truck Preference given to requirement of a four door crew cab model |
|  |  | Horizontal Bucket Reach | 30 to 60 feet (depending on model) |
|  |  | Working Height | 45ft working height for small truck, |
|  |  | Platform (Bucket) Capacity | Up to 800 lbs Two man bucket with tools |
|  |  | Engine Type | Diesel |
|  |  | Drive Configuration | 4x4 |
|  |  | Drive | LHD Left Hand Drive |
|  |  | Transmission | Automatic |
|  |  | Outrigger System | Hydraulic outriggers for stability with pads |
|  |  | Insulated Aerial Device | - Fiberglass-reinforced polymer (FRP) insulated boom sections and bucket - Dielectric rating: minimum 15 kV phase-to-ground working voltage - Tested and certified to ANSI/SAIA A92.2 electrical insulation standards - Dielectric test certificate provided at delivery with test voltage and date - Annual dielectric re-test capability documentation provided - Permanent voltage rating decals on boom and bucket - Bucket shall include dielectric floor mat and insulated tool tether points - Supplier shall provide training on annual re-testing procedures and safety requirements |
|  |  | Boom Type | Articulated over center or over rear wheels |
|  |  | Boom Rotation | 360 degrees |
|  |  | Control System | Upper and Lower control stations |
|  |  | Power Take-Off (PTO) | Hydraulic PTO |
|  |  | Safety Features | Emergency lowering system, harness attachments, anti-slip platform |
|  |  | Lighting System | Work and night-vision lighting |
|  |  | Tool Storage | Full Utility body complete with Built-in tool compartments and storage areas including hotstick compartment and safety rails and access steps. |
|  |  | Weather Resistance | All-weather operation with corrosion-resistant components |
|  |  | Additional Features | Rearview cameras, telematics systems Backup audible alarm system |
|  |  | Training | At least two days of in-country training in RMI for MEC employees on SOPs, equipment, safety, and maintenance. |
|  |  | Factory Acceptance Test (FAT) | Including: - Structural Tests: Inspecting the frame, boom, outriggers, and other critical components for structural integrity. This may involve visual inspections and potentially load tests. - Functional and Operational Tests: Verifying that all controls, hydraulics, and safety systems function correctly. This includes checking the bucket's movement, outriggers, brakes, lights, and alarms. - Dielectric Tests: Assessing the electrical insulation of the truck's components, particularly important for insulated bucket trucks. This test helps ensure the truck can safely operate around electrical lines. - Documentation and Sign-Off: The FAT should be well-documented, including a punch list of any issues found and a final sign-off from both the manufacturer and the customer. |
|  |  | Lifting Charts | The supplier must provide certified Lifting Charts. Plus a copy must be attached to the vehicle as a decal. |
|  |  | Documentation | Manual on standard vehicle operating procedures (SOPs) |
|  |  | Documentation | Parts and operation manuals for Arial lift system |
|  |  | Documentation | Parts and operation manuals for truck and engine |
|  |  | Standards | Must comply with ANSI/SAIA A92.2 Vehicle-Mounted Elevating and Rotating Aerial Devices |
|  |  | Truck Cab | Four door crew cab, air conditioned with side steps and full mirrors |
|  |  | Lighting systems | All LED lights |
|  |  | Hydraulic system | Hydraulic tool connectors at bucket With two flow controls and port reliefs to limit pressure |
|  |  | Tires | Full size spare tire on rim. |
|  |  | Fuel filters | 12 qty spare |
|  |  | Oil filters | 12 qty spare of each type (including hydraulic etc) |
|  |  | Fan belts | 2 complete sets of all belts for engine |
|  |  | Preferred color | White |
|  | Engine Start/Stop at Upper and lower control with emergency stop Rotation |  |  |
|  |  | Lifting Jib / Material handler and winch rated to 2000lbs. Complete with 75ft of ½” load line with swivel hook and safety latch |  |
|  |  | Hydraulic platform rotation and tilt |  |
|  |  | Boom Level |  |
|  |  | Built in multiple LED Spotlights and floodlights |  |
|  |  | Front Winch |  |
|  |  | Trailer Brake controls |  |
|  |  | 15 ton Pintle combo towing hitch with 2” stainless ball |  |
|  |  | Undersealing of complete vehicle |  |
|  |  | Front crash bars, industry quality, full width of front of vehicle |  |
|  |  | Inverter power system with multiple weather rated two 120V outlets on both sides of the rear body. |  |
|  |  | Full safety, warning, maintenance and operational decals |  |
|  |  | Heavy duty seat covers |  |
|  |  | Roof mounted strobe type emergency service flashing light bar |  |
|  |  | Earthing rod and cables |  |
|  |  | Rubber wheel chocks with eye bolts |  |
|  |  | Front and rear mudflaps |  |
|  |  | 7 prong trailer electrical connector plug |  |
|  |  | Fire extinguishers |  |
|  |  | Basic maintenance tool kit |  |
|  |  | Operating Fuel Specification: Automotive Diesel fuel (50ppm Max S) Cetane index min of 46 as per ASTM D976 Density @ 15 degs C 820.0 – 860.0 kg/m3 per ASTM D4052 Flash Point - PMCC min of 64.0 degs C as per ASTM D93 Viscosity @ 40 degs C of 2.0 – 4.5 CST as per ASTM D445 Lubricity, Max of 460 Micron as per I450 (full specification available upon request) |  |

Bucket Truck - Large

Delivery Location = Majuro Atoll Port, Marshall Islands – Qty = 1

Technical Specifications for Bucket Truck - Large

| Item | Specification | Basic Details |
| --- | --- | --- |
|  | Vehicle Type | Utility Bucket Truck Preference given to requirement of a four door crew cab model |
|  | Horizontal Bucket Reach | 30 to 60 feet (depending on model) |
|  | Working Height | 55ft working height for larger truck |
|  | Platform (Bucket) Capacity | Up to 800 lbs Two man bucket with tools |
|  | Engine Type | Diesel |
|  | Drive Configuration | 4x4 |
|  | Drive | LHD Left Hand Drive |
|  | Transmission | Automatic |
|  | Outrigger System | Hydraulic outriggers for stability with pads |
|  | Insulated Aerial Device | - Fiberglass-reinforced polymer (FRP) insulated boom sections and bucket - Dielectric rating: minimum 15 kV phase-to-ground working voltage - Tested and certified to ANSI/SAIA A92.2 electrical insulation standards - Dielectric test certificate provided at delivery with test voltage and date - Annual dielectric re-test capability documentation provided - Permanent voltage rating decals on boom and bucket - Bucket shall include dielectric floor mat and insulated tool tether points Supplier shall provide training on annual re-testing procedures and safety requirements |
|  | Boom Type | Articulated over center or over rear wheels |
|  | Boom Rotation | 360 degrees |
|  | Control System | Upper and Lower control stations |
|  | Power Take-Off (PTO) | Hydraulic PTO |
|  | Safety Features | Emergency lowering system, harness attachments, anti-slip platform |
|  | Lighting System | Work and night-vision lighting |
|  | Tool Storage | Full Utility body complete with Built-in tool compartments and storage areas including hotstick compartment and safety rails and access steps. |
|  | Weather Resistance | All-weather operation with corrosion-resistant components |
|  | Additional Features | Rearview cameras, telematics systems Backup audible alarm system |
|  | Training | At least two days of in-country training in RMI for MEC employees on SOPs, equipment, safety, and maintenance. |
|  | Factory Acceptance Test (FAT) | Including: - Structural Tests: Inspecting the frame, boom, outriggers, and other critical components for structural integrity. This may involve visual inspections and potentially load tests. - Functional and Operational Tests: Verifying that all controls, hydraulics, and safety systems function correctly. This includes checking the bucket's movement, outriggers, brakes, lights, and alarms. - Dielectric Tests: Assessing the electrical insulation of the truck's components, particularly important for insulated bucket trucks. This test helps ensure the truck can safely operate around electrical lines. - Documentation and Sign-Off: The FAT should be well-documented, including a punch list of any issues found and a final sign-off from both the manufacturer and the customer. |
|  | Lifting Charts | The supplier must provide certified Lifting Charts. Plus a copy must be attached to the vehicle as a decal. |
|  | Documentation | Manual on standard vehicle operating procedures (SOPs) |
|  | Documentation | Parts and operation manuals for Arial lift system |
|  | Documentation | Parts and operation manuals for truck and engine |
|  | Standards | Must comply with ANSI/SAIA A92.2 Vehicle-Mounted Elevating and Rotating Aerial Devices |
|  | Truck Cab | Four door crew cab, air conditioned with side steps and full mirrors |
|  | Lighting systems | All LED lights |
|  | Hydraulic system | Hydraulic tool connectors at bucket With two flow controls and port reliefs to limit pressure |
|  | Tires | Full size spare tire on rim. |
|  | Fuel filters | 12 qty spare |
|  | Oil filters | 12 qty spare of each type (including hydraulic etc) |
|  | Fan belts | 2 complete sets of all belts for engine |
|  | Preferred color | White |
|  | Engine Start/Stop at Upper and lower control with emergency stop Rotation |  |
|  | Lifting Jib / Material handler and winch rated to 2000lbs. Complete with 75ft of ½” load line with swivel hook and safety latch |  |
|  | Hydraulic platform rotation and tilt |  |
|  | Boom Level |  |
|  | Built in multiple Spotlights and floodlights |  |
|  | Front Winch |  |
|  | Trailer Brake controls |  |
|  | 15 ton Pintle combo towing hitch with 2” stainless ball |  |
|  | Undersealing of complete vehicle |  |
|  | Front crash bars, industry quality, full width of front of vehicle |  |
|  | Inverter power system with multiple weather rated two 120V outlets on both sides of the rear body. |  |
|  | Full safety, warning, maintenance and operational decals |  |
|  | Heavy duty seat covers |  |
|  | Roof mounted strobe type emergency service flashing light bar |  |
|  | Earthing rod and cables |  |
|  | Rubber wheel chocks with eye bolts |  |
|  | Front and rear mudflaps |  |
|  | 7 prong trailer electrical connector plug |  |
|  | Fire extinguishers |  |
|  | Basic maintenance tool kit |  |
|  | Operating Fuel Specification: Automotive Diesel fuel (50ppm Max S) Cetane index min of 46 as per ASTM D976 Density @ 15 degs C 820.0 – 860.0 kg/m3 per ASTM D4052 Flash Point - PMCC min of 64.0 degs C as per ASTM D93 Viscosity @ 40 degs C of 2.0 – 4.5 CST as per ASTM D445 Lubricity, Max of 460 Micron as per I450 (full specification available upon request) |  |